| Glacial Morphometry |  |
| --- | --- |
|  | ## Step1: Compute Glacier Area and Perimeter Input: glacier.geojson Output: glacierAP.geojson ### Algorithms - Read the input glacier GeoJSON file. - Reproject the glacier data to a projected CRS to ensure correct area and perimeter calculations. - Compute glacier area in square meters (m²). - Convert glacier area from m² to km². - Compute glacier perimeter in meters (m). - Convert glacier perimeter from meters to kilometers (km). - Round all computed area and perimeter values to three decimal places. - Save the output GeoJSON with the new attribute fields: - Area_m2 - Area_km2 - Perimeter_m - Perimeter_km ## Step2: Compute Ice Compactness Index (ICI) for Each Glacier Segment Input: glacierAP.geojson Output: glacierICI.geojson ### Algorithms - Read the glacier GeoJSON file containing area and perimeter attributes. - Ensure that Area and Perimeter columns exist in the dataset. - Merge (dissolve) all glacier polygons based on W_ID. - Recalculate Area and Perimeter for each merged Micro-watershed. - Compute the Ice Compactness Index (ICI) for each Micro-watershed using: - ICI = 4*Pi*Area / Perimeter2 - Save the output GeoJSON with the following attributes: - W_ID - Area_m2 - Perimeter_m - ICI ## Step3:Attached ICI Index with Micro-watershed Input: - glacierICI.geojson - watershed_UTista_selected_WID.geojson Output: glacierICI_W.geojson ### Algorithms - Read the glacier ICI GeoJSON file. - Read the Micro-watershed GeoJSON file. - Ensure both datasets use the same CRS. - Perform an attribute join using W_ID. - Retain the original Micro-watershed geometry and add the ICI-related attributes. - Save the merged dataset as glacierICI_W.geojson. |
|  | ## Step 1: Extract Maximum Glacier Region per Micro-Watershed Input: glacierAP.geojson Output: glacier_max.geojson ### Algorithms - Read the glacier GeoJSON file. - Ensure the CRS is projected for correct area computation in square meters. - Compute glacier area in m² and km². - Verify that the watershed identifier W_ID exists. - Identify the glacier polygon with the maximum area within each micro-watershed. - Save the output GeoJSON with the following attributes: - W_ID - Area_m2 - Area_km2 - geometry ## Step 2: Extract Elevation for Each Vertex of Glacier Geometric Region Input: - GLOFDEM.tiff - glacier_max.geojson Output: vert_elev_glacier.geojson ### Algorithms - Read the glacier GeoJSON file. - Open the DEM (GLOFDEM.tiff). - Loop through each glacier polygon. - Extract watershed ID and glacier ID. - Handle MultiPolygon geometries. - Extract vertices from the exterior ring of each polygon. - Sample DEM elevation at each vertex location. - Create a GeoDataFrame containing vertex geometry and elevation. - Save the output as vert_elev_glacier.geojson. ## Step 3: Extract Minimum (Snout) and Maximum Glacier Elevation Points Input: vert_elev_glacier.geojson Outputs: - snout_points.geojson - max_glacier_points.geojson ### Algorithms - Read the glacier vertex–elevation GeoJSON. - Remove invalid or missing elevation values. - Calculate the minimum elevation (snout elevation) per watershed. - Save the snout elevation points. - Calculate the maximum glacier elevation per watershed. - Save the maximum glacier elevation points. ## Step 4: Extract Elevation for Each Vertex of Micro-Watershed Geometry Input: - GLOFDEM.tiff - watershed_UTista_selected_WID.geojson Output: vert_elev_watershed.geojson ### Algorithms - Read the watershed GeoJSON file. - Open the DEM (GLOFDEM.tiff). - Loop through each watershed feature. - Handle different geometry types. - Extract coordinates from each geometry: - Polygon → use exterior coordinates - Sample DEM elevation at each vertex. - Create a GeoDataFrame with vertex geometry and elevation. - Save the output as vert_elev_watershed.geojson. ## Step 5: Extract Minimum and Maximum Elevation Points for Micro-Watershed Input: vert_elev_watershed.geojson Outputs: - min_watershed_points.geojson - max_watershed_points.geojson ### Algorithms - Read the watershed vertex–elevation GeoJSON. - Remove invalid or missing elevation values. - Calculate the minimum watershed elevation per watershed. - Save the minimum elevation points. - Calculate the maximum watershed elevation per watershed. - Save the maximum elevation points. ## Step 6: Print Attribute Columns Inputs: - watershed_UTista_selected_WID.geojson - snout_points.geojson - max_glacier_points.geojson - min_watershed_points.geojson ### Algorithms - Read all input GeoJSON files. - Use a function to print attribute columns only. - Print attribute columns for each dataset. ## Step 7: Compute Snout Index (SI) Inputs: - watershed_UTista_selected_WID.geojson - snout_points.geojson - max_glacier_points.geojson - min_watershed_points.geojson Output: snout_index.geojson ### Algorithms - Read all input GeoJSON files. - Ensure CRS consistency across datasets. - Retain only required attribute fields. - Merge datasets using W_ID. - Compute Snout Index (SI) using: SI=Esnout−Emin / Emax−Emin - Save the output as snout_index.geojson. |
|  | ## Step 1: Extract Maximum Glacier Region per Micro-Watershed Input: glacierAP.geojson Output: glacier_max.geojson ### Algorithms - Read the glacier GeoJSON file. - Ensure the CRS is projected for accurate area computation in square meters. - Compute glacier area in m² and km². - Verify that the watershed identifier W_ID exists. - Identify the glacier polygon with the maximum area within each micro-watershed. - Save the output GeoJSON with the following attributes: - W_ID - Area_m2 - Area_km2 - geometry ## Step 2: Extract Equilibrium Line Altitude (ELA) Contour Line per Micro-Watershed Input: - GLOFDEM.tiff - glacier_max.geojson - watershed_UTista_selected_WID.geojson Output: ELA_lines_longest.geojson ### Algorithms - Load the DEM, glacier maximum area polygons, and watershed GeoJSON files. - Fix invalid geometries where required. - Ensure all datasets use the same CRS. - Define a function to compute ELA from the DEM. - Define a function to extract ELA contour lines. - Retain only the longest ELA contour line for each micro-watershed. - Iterate through each watershed using the actual W_ID. - Save the extracted longest ELA contour lines as a GeoJSON file. ## Step 3: Identify Snow Accumulation Zone (SAZ) Input: - GLOFDEM.tiff - glacier_max.geojson - ELA_lines_longest.geojson Output: SAZ_all_glaciers.geojson ### Algorithms - Read all input datasets. - Merge ELA values into glacier polygons using W_ID. - Read the DEM and mask NoData values. - Ensure glacier polygons match the DEM CRS. - Process each glacier polygon individually: - Mask the DEM using the glacier polygon. - Create a Snow Accumulation Zone (SAZ) mask. - Convert the SAZ mask to polygon features. - Combine SAZ polygons for all glaciers. - Save the output as SAZ_all_glaciers.geojson. ## Step 4: Extract Maximum Snow Accumulation Zone per Micro-Watershed Input: SAZ_all_glaciers.geojson Output: Accumulation_glaciers_max area.geojson ### Algorithms - Read the SAZ GeoJSON file. - Check and fix the CRS if required. - If the CRS is geographic (degrees), reproject to a projected CRS (e.g., UTM). - Compute the area of each SAZ polygon. - Extract the maximum area SAZ polygon per micro-watershed using W_ID. - Save the output GeoJSON. ## Step 5: Extract Entire Glacier Region per Micro-Watershed Input: glacier.geojson Output: glaciers_Entirearea.geojson ### Algorithms - Read the glacier GeoJSON file. - Check and fix the CRS. - If the CRS is geographic (latitude/longitude), reproject to a projected CRS suitable for area computation. - Merge (dissolve) glacier polygons by W_ID to represent the entire glacier region per micro-watershed. - Save the output GeoJSON. - Confirm that glacier polygons are successfully merged by watershed. ## Step 6: Compute Glacier Active Accumulation Ratio Input: - watershed_UTista_selected_WID.geojson - Accumulation_glaciers_max area.geojson - glaciers_Entirearea.geojson Output: accumulation_ratio.geojson ### Algorithms - Read all input GeoJSON files. - Ensure CRS consistency across all datasets. - Retain only non-geometry attribute columns required for joining. - Rename area columns for clarity prior to merging. - Merge all datasets using the watershed identifier W_ID. - Compute the Accumulation Area Ratio (AAR / AAA). - Save the output GeoJSON with the following attributes: - W_ID - Accum_Area_sqm - Glacier_Area_sqm - AAR |
|  | ## Step 1: Intersection of Micro-Watershed Region with Glacier Region and Area Computation Input: - watershed_UTista_selected_WID.geojson - glacier_max.geojson Output: watershed_glacier_area.geojson ### Algorithms - Read the Micro-watershed boundary GeoJSON file. - Read the maximum glacier coverage GeoJSON file. - Check whether both layers use the same CRS. - If the CRS differs, reproject the glacier layer to match the CRS of the Micro-watershed layer. - Rename the watershed identifier field to prevent naming conflicts: - W_ID → WID_MAIN. - Compute the area of each micro-watershed in square meters (sqm). - Perform a spatial intersection between the glacier layer and the micro-watershed layer. - Compute the glacier area for each intersected micro-watershed segment. - Group the segmented glacier regions by WID_MAIN. - Sum the glacier area values for each micro-watershed. - Merge the glacier segments accordingly for each watershed. - Rename the watershed identifier field back to: - WID_MAIN → W_ID. - Save the output GeoJSON as watershed_glacier_area.geojson. ## Step 2: Compute Glacier–Watershed Area Ratio (GWAR) Input: watershed_glacier_area.geojson Output: WGR.geojson ### Algorithms - Read the input GeoJSON file. - Check that all required columns exist. - Compute the Glacier–Watershed Area Ratio (GWAR) as: GWAR=Glacier Area/Watershed Area - Optionally replace NaN or infinite values with zero. - Save the output GeoJSON as WGR.geojson |
|  | ## Algorithm: Glacier–Watershed Length–Width Parameters & GLWI (GEQ) ### Step 1: Assignment of Multiple Width Lines for Each Micro-Watershed Input: watershed_UTista_selected_WID.geojson Output: watershed_width_WID.geojson - Read micro-watershed boundary file - Ensure CRS is projected (meters) - Define parameters - Number of points along main axis - Length of perpendicular width lines (meters) - Extract maximum length (main axis) line - Generate perpendicular width lines along the axis - Intersect width lines with watershed polygon - Select longest intersection as width at each position - Compute mean width per watershed - Store results: W_ID, axis line, width lines, mean width ### Step 2: Extraction of Glacier Width Lines per Micro-Watershed Input: watershed_width_WID.geojson, Output: watershed_width_clipped_glacier.geojson glacier_max.geojson - Read watershed width lines and glacier polygons - Ensure CRS consistency (meters) - Overlay (intersection) width lines with glacier polygons - Retain only line segments inside glaciers ### Step 3: Compute Mean Width for Micro-Watershed Input: watershed_width_WID.geojson Output: watershed_widthmean.geojson - Read width line data - Compute length of each width line - Calculate mean width per W_ID - Attach mean width to lines or keep one representative line ### Step 4: Compute Mean Width for Glacier Input: watershed_width_clipped_glacier.geojson Output: glacier_widthmean.geojson - Read clipped glacier width lines - Compute length of each line - Calculate mean glacier width per W_ID - Attach mean width or retain representative line ### Step 5: Compute Glacier Length per Micro-Watershed Input: Wcenterline_length.geojson, glacier_max.geojson Output: watershed_length_clipped_glacier.geojson - Read glacier centerline and glacier polygon - Ensure CRS consistency - Overlay (intersection) centerline with glacier polygon - Clip centerline segments by W_ID ### Step 6: Compute Total Glacier Length Input: watershed_length_clipped_glacier.geojson Output: watershed_length_clipped_glacier_WID_total_length.geojson - Read clipped glacier centerlines - Ensure CRS is in meters - Dissolve lines by W_ID - Compute total glacier length per watershed ### Step 7: Attach Length and Width Parameters to Micro-Watersheds Inputs: - watershed_UTista_selected_WID.geojson - watershed_length_clipped_glacier_WID_total_length.geojson - glacier_widthmean.geojson - Wcenterline_length_computed.geojson - watershed_widthmean.geojson - Read micro-watershed polygon (base layer) - Read glacier length and glacier mean width - Read watershed length and watershed mean width - Standardize W_ID in all datasets - Join all parameters to watershed polygon - Keep required attributes only Output: watershed_L_W_parameters_WID.geojson Columns: W_ID, Glacial_Len, Glacial_Width, Watershed_Len, Watershed_Width ### Step 8: Join Elevation Parameters Inputs: - watershed_L_W_parameters_WID.geojson - snout_points.geojson - max_watershed_points.geojson - Read base length–width parameter file - Read snout elevation points - Read maximum elevation points - Join elevation attributes using W_ID - Handle missing elevation values if required Output: watershed_L_W_parameters_ELEV_WID.geojson Final Columns: W_ID, Glacial_Len, Glacial_Width, Watershed_Len, Watershed_Width, geometry, Snout_Elev_m, Max_Elev_m ### Step 9: Compute Glacier Length–Width Index (GLWI / GEQ) Input: watershed_L_W_parameters_WID.geojson - Read glacier length–width data - Ensure numeric values - Compute GEQ - GEQ = Glacial_Len / Glacial_Width - If Glacial_Len = 0 or Glacial_Width = 0 → GEQ = 0 - Keep required columns only Output: glacier_L_W_GEQ.geojson Columns: W_ID, Glacial_Len, Glacial_Width, EQ |
|  | ## Compute Glacier Slope Index (GSLI) Input: watershed_L_W_parameters_ELEV_WID.geojson Output: glacier_L_W_GSLI.geojson ### Algorithms - Read the input GeoJSON file (watershed_L_W_parameters_ELEV_WID.geojson). - Ensure all required fields are numeric and handle missing values appropriately. - Compute the Glacier Slope Index (GSLI) using: GSLI=MaxE−SnoutElev / GlacialLen - Apply the rule: - If GlacialLen ≥ 0.1, then GSLI = 0. - Remove any negative GSLI values, if present. - Retain only the required attribute columns: - W_ID - Glacial_Len - Max_Elev_m - Snout_Elev_m - GSLI - Save the output as a GeoJSON file (glacier_L_W_GSLI.geojson). |
|  | ## Compute Glacier GWPQI & GLPQI Input: watershed_L_W_parameters_WID.geojson Output: glacier_L_W_PQI.geojson ### Algorithms - Read the input GeoJSON file (watershed_L_W_parameters_WID.geojson). - Identify and verify required column names for glacier and watershed length and width. - Compute global length ratio: globallengthratio = total_gl_len / total_ws_len - Compute global width ratio: globalwidthratio = total_gl_wid / total_ws_wid - Check for missing values:If any glacial attribute is NULL, exclude the record from PQI computation. - Initialize output columns: PQI_Length, PQI_Width - Compute PQI_Length for valid rows: PQI_Length = (GL_LEN / WS_LEN) / globallengthratio - Compute PQI_Width for valid rows: PQI_Width = (GL_W / WS_W) / globalwidthratio - Retain only valid (non-NULL) rows used in PQI computation. - Verify PQI values are computed correctly for both length and width. - Save the final output as GeoJSON (glacier_L_W_PQI.geojson). |

| Glacier Lake Morphometry |  |
| --- | --- |
|  | ## Step 1: Compute Glacier Lake Area and Perimeter ### Input: glacier_lake.geojson ### Output: glacier_lakeAP.geojson ### Algorithm - Read glacier lake GeoJSON file. - Check coordinate reference system (CRS). - Reproject glacier lake polygons to a projected CRS (meters) for accurate area and perimeter computation. - Compute glacier lake area in square meters (m²). - Convert area from m² to square kilometers (km²). - Compute glacier lake perimeter in meters (m). - Convert perimeter from meters to kilometers (km). - Round area and perimeter values to three decimal places. - Add new attribute columns: - Area_m2 - Area_km2 - Perimeter_m - Perimeter_km - Save the updated dataset as GeoJSON. ## Step 2: Compute Shoreline Development Index (SDI) for Each Glacier Lake Segment ### Input: glacier_lakeAP.geojson ### Output: glacier_lakeSDI.geojson ### Algorithm - Read glacier lake area–perimeter GeoJSON. - Ensure Area_m2 and Perimeter_m columns exist. - Merge (dissolve) all glacier lake polygons by W_ID. - Recalculate total area and perimeter for each merged micro-watershed. - Compute Shoreline Development Index (SDI) using: SDI=Perimeter_m2×π×Area_m2SDI = \frac{Perimeter\_m}{2 \times \sqrt{\pi \times Area\_m2}}SDI=2×π×Area_m2​Perimeter_m​ - Attach SDI values to corresponding W_ID. - Retain required attributes: - W_ID - Area_m2 - Perimeter_m - SDI - Save the output as GeoJSON. ## Step 3: Attach SDI with Micro-Watershed ### Input: glacier_lakeSDI.geojson watershed_UTista_selected_WID.geojson ### Output: glacier_LakeSDI_W.geojson ### Algorithm - Read glacier lake SDI GeoJSON. - Read micro-watershed boundary GeoJSON. - Ensure both datasets have the same CRS. - Perform attribute join using W_ID. - Preserve original watershed geometry. - Attach SDI and glacier lake metrics to micro-watershed polygons. - Retain required attributes: - W_ID - Area_m2 - Perimeter_m - SDI - Save merged dataset as GeoJSON. |
|  | ## Step 1: Intersect Glacial Lake with Micro-Watershed and Compute Area ### Input watershed_UTista_selected_WID.geojson glacier_lake.geojson ### Output watershed_glacierlake_area.geojson ### Algorithm - Read micro-watershed boundary GeoJSON. - Read glacier lake GeoJSON. - Check that both layers use the same coordinate reference system (CRS). - If CRS differs, reproject glacier lake layer using the micro-watershed CRS as reference. - Rename watershed identifier to avoid naming conflict: - "W_ID" → "WID_MAIN". - Compute micro-watershed area in square meters (sqm). - Perform spatial intersection between glacier lake polygons and micro-watershed polygons. - Compute glacier lake area for each intersected polygon segment. - Group intersected glacier lake segments by WID_MAIN. - Sum glacier lake area for each micro-watershed. - Merge glacier lake segments accordingly. - Rename watershed identifier back: - "WID_MAIN" → "W_ID". - Save the result as GeoJSON. ## Step 2: Compute Glacier Lake–Watershed Area Ratio (GLWAR) ### Input watershed_glacierlake_area.geojson ### Output WGLR.geojson ### Algorithm - Read watershed_glacierlake_area.geojson. - Check that required columns exist: - Glacier lake area - Watershed area - Compute Glacier Lake–Watershed Area Ratio (GLWAR) using: GLWAR=Glacier lake area / Micro-watershed area - Replace NaN or infinite values with 0 (optional). - Save the output GeoJSON as WGLR.geojson. |
|  | ## Step 1: Intersect Micro-Watershed Centerline with Glacier Lake ### Input glacier_lake.geojson centerline_smoothed_full.geojson ### Output lake_centerline_length_WID.geojson ### Algorithm - Read glacier lake GeoJSON. - Read micro-watershed centerline GeoJSON. - Keep only required attribute fields. - Reproject both layers to a metric CRS. - Perform spatial intersection between centerline and glacier lake polygons. - Compute length of each intersected line segment. - Dissolve intersected line segments by W_ID, keeping line geometry. - Sum all seg_len_m values for each W_ID. - Save the output as GeoJSON. ## Step 2: Intersect Micro-Watershed Width with Glacier Lake ### Input glacier_lake.geojson watershed_width_WID.geojson ### Output lake_width_clipped.geojson ### Algorithm - Read glacier lake GeoJSON. - Read micro-watershed width GeoJSON. - Keep only required attribute fields. - Reproject both layers to a metric CRS. - Perform spatial intersection between width lines and glacier lake polygons. - Identify the correct W_ID column. - Standardize the watershed identifier column name. - Retain only required fields. - Save the output as GeoJSON. ## Step 3: Compute Mean Lake Width per Micro-Watershed ### Input lake_width_clipped.geojson ### Output lake_mean_width_WID.geojson ### Algorithm - Read clipped lake width GeoJSON. - Ensure the dataset is in a metric CRS. - Compute length of each width line. - Calculate mean width for each W_ID. - Create a representative geometry (one feature per W_ID). - Save the output as GeoJSON. ## Step 4: Extract Minimum and Maximum Lake Elevation ### Input glacier_lake.geojson DEM ### Output lake_min_max_elevation_WID.geojson ### Algorithm - Read glacier lake polygons. - Convert lake polygons into vertex points. - Read DEM. - Sample elevation values at each vertex. - Remove invalid elevation values. - Compute minimum and maximum elevation per W_ID. - Assign representative lake geometry. - Retain attributes: - Lake_Min_Elev_m - Lake_Max_Elev_m - Save the output as GeoJSON. ## Step 5: Join All Lake and Micro-Watershed Parameters ### Input - watershed_UTista_selected_WID.geojson - Wcenterline_length_computed.geojson - watershed_widthmean.geojson - lake_centerline_length_WID.geojson - lake_mean_width_WID.geojson - lake_min_max_elevation_WID.geojson ### Output glacial_lake_L_W_all_parameters.geojson ### Algorithm - Read micro-watershed boundary GeoJSON. - Read micro-watershed length table. - Read micro-watershed width table. - Read lake length table. - Read lake mean width table. - Read lake elevation table. - Standardize attribute names across all datasets. - Merge all attribute tables using W_ID. - Handle missing or null values. - Save final merged dataset as GeoJSON. ## Step 6: Compute Glacier Lake Elongation Ratio (GLEQ) ### Input glacial_lake_L_W_all_parameters.geojson ### Output glacial_lake_L_W_GLEQ.geojson ### Algorithm - Read glacier lake length–width parameter GeoJSON. - Ensure all required fields are numeric. - Compute Glacier Lake Elongation Ratio (GLEQ) using: GLEQ=Lake_Length_m/ Lake_Mean_Width_m - Apply rule: - If Lake_Length_m == 0 or Lake_Mean_Width_m == 0, set GLEQ = 0. - Retain required attributes: - W_ID - Lake_Length_m - Lake_Mean_Width_m - GLEQ - geometry - Save the output as GeoJSON. |
|  | ## Compute Lake Slope Index (GLSI) Input file: glacial_lake_L_W_all_parameters.geojson Output file: glacial_lake_L_W_GLSI.geojson ### Algorithms - Read Input Data Read the GeoJSON file glacial_lake_L_W_all_parameters.geojson. - Ensure Numeric Values & Handle Missing Data Convert required columns to numeric values. Handle missing or invalid values appropriately. - Check Required Columns Exist Ensure the following fields are present: - W_ID - Lake_Length_m - Lake_Min_Elev_m - Lake_Max_Elev_m - Initialize GLSI Column Create a new column named GLSI. - Apply Length Rule - If Lake_Length_m < 0.1 → set GLSI = 0 - If Lake_Length_m ≥ 0.1 → GLSI is eligible for computation - Compute Lake Longitudinal Slope Index (GLSI) Use the equation: GLSI=Lake_Max_Elev_m−Lake_Min_Elev_mLake_Length_mGLSI = \frac{Lake\_Max\_Elev\_m - Lake\_Min\_Elev\_m}{Lake\_Length\_m}GLSI=Lake_Length_mLake_Max_Elev_m−Lake_Min_Elev_m​ - Remove Negative GLSI Values If any computed GLSI value is negative, replace it with 0. - Keep Only Required Output Columns Retain the following fields only: - W_ID - Lake_Length_m - Lake_Min_Elev_m - Lake_Max_Elev_m - GLSI - Save Output File Save the final GeoJSON as: glacial_lake_L_W_GLSI.geojson |
|  | ## Compute Glacial Lake Length–Width PQI (GLLPQI & GLWPQI) Input file: glacial_lake_L_W_all_parameters.geojson Output file: glacial_lake_L_W_PQI.geojson ### Algorithms - Read Input Data Read the GeoJSON file glacial_lake_L_W_all_parameters.geojson. - Display and Verify Column Names (Lake & Watershed) Ensure required columns for: - Lake Length (LK_LEN) - Lake Width (LK_WID) - Watershed Length (WS_LEN) - Watershed Width (WS_WID) - Compute Global Ratios (Ignore NULL Values) - Compute total lake length: total_lk_len = sum of all LK_LEN - Compute total watershed length: total_ws_len = sum of all WS_LEN - Compute total lake width: total_lk_wid = sum of all LK_WID - Compute total watershed width: total_ws_wid = sum of all WS_WID - Compute Global Length and Width Ratios - global_len_ratio = total_lk_len / total_ws_len - global_wid_ratio = total_lk_wid / total_ws_wid - Strict NULL Check - If any lake-related attribute is NULL, do not compute PQI for that record. - Initialize Output Columns - LPQI_Length - LPQI_Width - Select Valid Rows Only Valid rows must satisfy: - LK_LEN > 0 - LK_WID > 0 - WS_LEN > 0 - WS_WID > 0 - global_len_ratio > 0 - global_wid_ratio > 0 - Compute LPQI for Valid Rows - LPQI_Length LPQILength=(LK_LEN/WS_LEN) / global_len_ratio - LPQI_Width LPQIWidth=(LK_WID/WS_WID) / global_wid_ratio​ - Zero Rule - If either LPQI_Length or LPQI_Width is zero, then set both to zero. - Save Output Save the final GeoJSON as: glacial_lake_L_W_PQI.geojson |

| Catchment Morphometric parameters |  |
| --- | --- |
|  | ## Step 1: Extract Elevation Statistics for Micro-Watershed ### Input - Micro-watershed polygon (MWatershed) - DEM ### Output - Watershed_catchment_relief.geojson ### Algorithm - Read micro-watershed polygon dataset. - Read DEM. - Check CRS of micro-watershed and DEM. - If CRS does not match DEM, reproject micro-watershed to DEM CRS. - Perform zonal statistics on DEM for each micro-watershed. - Extract elevation statistics: - Minimum elevation - Maximum elevation - Mean elevation - Store extracted elevation statistics with corresponding W_ID. - Save the output as GeoJSON. ## Step 2: Compute Hypsometric Index (HI) ### Input - Watershed_catchment_relief.geojson ### Output - Catchemnt_Hypsometric_Index.geojson ### Algorithm - Read Watershed_catchment_relief.geojson. - Select required columns: - W_ID - Mean_Elev - Min_Elev - Max_Elev - Compute Hypsometric Index (HI) using: HI=Mean_Elev−Min_Elev / Max_Elev−Min Elev ​ - Save the output as GeoJSON. |
|  | ## Compute Relief Ratio (RR) Input: Watershed_catchment_relief.geojson Output: catchment_relief_ratio.geojson ### Algorithms - Read the catchment GeoJSON file (Watershed_catchment_relief.geojson). - Reproject the catchment geometry to UTM Zone 45N to ensure accurate metric calculations. - Compute the area of the catchment in square meters (m²). - Compute the Relief Ratio (RR) using: RR={(MaxE−MinE) / Area}×100 - Select only the required output attribute fields. - Save the output GeoJSON as catchment_relief_ratio.geojson. |
|  | ## Step 1: Create Centreline of Micro-Watershed ### Input DEM Micro-watershed polygon (MWatershed) ### Output centerlines_final.geojson ### Algorithm - Read DEM and micro-watershed polygon. - Create a polygon mask for each micro-watershed. - Convert polygon mask to raster mask. - Apply skeletonization on the mask (mask > 0). - If skeleton is too small, apply fallback method. - Convert skeleton pixels to geographic coordinates. - Sort skeleton coordinates by distance to approximate the centreline. - Mask DEM with the watershed polygon. - Generate centreline for each micro-watershed. - Attach corresponding W_ID. - Save all centrelines as GeoJSON. ## Step 2: Extract Vertex Elevation of Centreline ### Input centerlines_final.geojson DEM ### Output Vertex elevation dataset (GeoJSON) ### Algorithm - Read centreline GeoJSON. - Read DEM. - Extract vertices from centreline geometry: - Case 1: LineString - Case 2: MultiLineString - Create a vertex GeoDataFrame. - Reproject vertices to DEM CRS if required. - Prepare coordinate list from vertex points. - Sample DEM elevation at each vertex. - Attach elevation values to vertices. - Save vertex elevation dataset. ## Step 3: Create LineString from Ordered Elevation Points ### Input centerline_vertices_elevation.geojson ### Output centerline_ordered_by_elevation.geojson ### Algorithm - Read vertex elevation GeoJSON. - Process vertices watershed-wise using W_ID. - Convert vertex coordinates and elevations to NumPy arrays. - Identify the lowest elevation point. - Compute full distance matrix between points. - Iteratively select the nearest point with higher elevation. - Exclude points with lower elevation. - Append selected points sequentially. - Construct LineString from ordered points. - Save the ordered centreline as GeoJSON. ## Step 4: Extend Centreline to Watershed Boundary ### Input centerline_ordered_by_elevation.geojson Micro-watershed polygon ### Output centerline_extended.geojson centerline_extended_corrected.geojson ### Algorithm - Read ordered centreline GeoJSON. - Read micro-watershed polygon. - Identify watershed containing each centreline (using midpoint). - Extend centreline to intersect watershed boundary. - Smooth the extended centreline. - Save extended centreline. - Apply geometric correction. - Save corrected centreline as GeoJSON. ## Step 5: Measure Centreline Length ### Input Extended centreline GeoJSON ### Output Centreline length values (attribute) ### Algorithm - Read centreline file. - Reproject to UTM Zone 45N (meters). - Measure centreline length in meters. - Store length as an attribute. ## Step 6: Compute Basin Length per Micro-Watershed ### Input centerline_smoothed_full.geojson watershed_UTista_selected_WID.geojson ### Output watershed_with_basin_length.geojson ### Algorithm - Read smoothed centreline GeoJSON. - Read micro-watershed boundary GeoJSON. - Reproject both layers to UTM Zone 45N. - Group centreline segments by W_ID. - Sum length of all line segments per W_ID. - Join basin length values to watershed polygons. - Save output GeoJSON. ## Step 7: Join Elevation and Basin Length Parameters ### Input Watershed_Elevation_Area_Stats.geojson watershed_with_basin_length.geojson ### Output Watershed_catchment_relief.geojson ### Algorithm - Read watershed elevation–area statistics file. - Read watershed basin length file. - Verify presence of W_ID in both datasets. - Select only W_ID and basin length column. - Join elevation and basin length datasets using W_ID. - Save combined GeoJSON. ## Step 8: Compute Catchment Slope Index (CSI / CSR) ### Input Watershed_catchment_relief.geojson ### Output catchment_slope_CSI.geojson ### Algorithm - Read catchment relief GeoJSON. - Reproject to UTM Zone 45N for accurate length measurement. - Use basin length (basin_length_m) as catchment length. - Compute Catchment Slope Index (CSI): CSI=MaxE−MinE / basin_length - Select required output fields. - Save output GeoJSON. |
|  | ## Compute Terrain Ruggedness Index (TRI) Input: Watershed_catchment_relief.geojson Output: catchment_terrain_ruggedness.geojson ### Algorithms - Read the catchment GeoJSON file (Watershed_catchment_relief.geojson). - Reproject the catchment geometry to UTM Zone 45N to ensure accurate metric calculations. - Compute the area of the catchment in square meters (m²). - Compute the Terrain Ruggedness Index (TRI) using: TRI= ((MaxE−MinE) / sqrt Area)×100 - Select only the required output attribute fields. - Save the output GeoJSON as catchment_terrain_ruggedness.geojson. |
|  | ## Compute Basin Form Factor (BFF) Input: Watershed_catchment_relief.geojson Output: catchment_form_factor_index.geojson ### Algorithms - Read the catchment GeoJSON file (Watershed_catchment_relief.geojson). - Reproject the catchment geometry to UTM Zone 45N to ensure accurate length measurement. - Compute the area of the catchment in square meters (m²). - Compute the Basin Form Factor (BF) using: BF=Area / (Length)2​ - Select only the required output attribute fields. - Save the output GeoJSON as catchment_form_factor_index.geojson. |